DSP Code v3.2.1

DSP Code 3 project (shortly DSPC3) is aimed at improving upon capabilities of Flowstone stock Code Component. It is loosely based on C and focuses on generating SSE compatible high-performance assembler code.

# compatibility with stock DSP CODE

Code from original code component will not be fully compatible with DSPC3. It can however be easily edited to work.

1. codeblocks( code written in "{}" brackets) must be terminated with ";" Codeblock is equivalent to comma operator in C. Most operators will assume the statement continues after "}"

2.Reading/writing arrays no longer uses float number as an index, but assumes input is in integer format. You have to implicitly convert float values to integer using "to_i(x)" function.

# syntax

As mentioned, syntax is loosely based on original Code component, C language and Ruby. However, one can say it is "closer to metal" than any of them. Specifically, it uses different operators for float and integer operations and also supports multiple types of statements, some SSE compatible, others not.

Here is a list of operators by order of execution. If not stated otherwise, the operation is floating point.

| operator | meaning |
| --- | --- |
| array[] array.[] () func() {} statement(){} | access array element, access array element (mono index), brackets, function call, codeblock, statement |
| ** | power operator ( alias for pow(x,y) ) |
| * .* / % | multiply, integer multiply (warning - not tested), divide, modulo |
| + - .+ .- | add, subtract, integer add, integer subtract |
| >> << | logical bitshift left, logical bitshift right |
| < <= > >= | comparison operators (they also work for integers, if at least one of them is positive) |
| == != | equal, not equal (also work for integers) |
| &! | bitwise AND NOT (second operator is inverted) |
| & | bitwise AND |
| ^ | bitwise XOR |
| ¦ | bitwise OR |
| condition ? optTRUE : optFALSE | ternary operator |
| = | assignment (note: assignment has lowest priority on its left side and highest on its right side) |

## formats

Two formats are supported: 32bit integer and 32bit(single precision) float. As mentioned, DSPC3 does not automatically convert between them. Constant written in the code is assumed to be float. Integer constant may be initialized in several ways.

streamin in; streamout out; float x=5; int y=5; stage(0){ //using contants in code 95.5; //float to_i(95.5); //integer 96 trunc(95.5); //integer 95 95i; //integer(decimal) 075; //integer(octal)=61 0x7a; //integer hexadecimal 0b011001; //integer binary !DOES NOT WORK (YET)! //converting values between formats to_i(x); //float to integer (with rounding) trunc(x); //float to integer (with truncation) to_f(y); //int to float };

There is also 3rd "hidden" format, which is binary mask. Helpful tip is that zero (in both int and float format) is equivalent to "FALSE" and integer -1 ( "-1i") is equivalent to "TRUE".

DSPC3 also supports arrays of floats. Integer arrays are not directly supported, though the format really matters only on initialization. You can put integers into the arrays just like floats. As mentioned before, unlike in stock Code component, when writing/reading element of an array, the index must be provided in int format. There are also two distinct operators to do that.

WARNING: unlike in Code Component, in DSPC3 the index does not get automatically clipped to range of the array <0..arraysize-1>. You need to do this manually if you suspect such situation may occur.

streamin index; memin memoryBlock[1024]; //memin initializes an array //with contents identical to mem float array1[256]=1; //you can optionally initialize array //with constant value float array2[100]=rand(-1,1); //or with random values from range array1[to_i(index)]=1.5; //remember, index argument must be integer array1.[to_i(index)]=0.5; // array.[] is a mono version of array[] //it uses index from sseChannel0 as index for //all 4 channels. It is significantly faster on CPU //use it only in cases when index for all 4 channels //is bound to be same. //it means .[] is not fully poly/mono4-compatible. array2[14]=1; //when index is constant .[] is used under the hood //automatically to reduce CPU load

## Operators

All operators you're familiar in Code component work in DSPC3 too. However, their functionality and flexibility is highly extended. Firstly, you are no longer limited by complexity of your expressions (it used to be, that using more than 8 consecutive operations produced faulty assembler code). Secondly, assignment ( "=" ) does not necessarily need to be at the start of each line. You can also assign values in middle of expressions. Be aware though - operations in each expression don't necessarily happen in the order they appear in the line. This may cause ambiguity when use are using value of variable, that is assigned to somewhere in the same expression.

Also Codeblock "{}" was added. Codeblock may contain multiple statements and returns the value of the last one. Effectively it works similar to comma operator in C.

streamin in; streamout out; float a,b,c; float frac; int index; a=b=c=1; //assigns 1 to c, then to b, then to a. //following expression splits "in" into integer and fractional part frac=in-to_f(index=trunc(in)); //codeblocks only return value of the last statement a={b=1; //first b is given value 1 a+b; //notice this line doen't even have assignment operator } //a is given (a+b) ~the last statement in the codeblock

DSPC3 also introduces few new operators. They have already been mentioned in the operator-precedence-table. Namely the power operator (a**b), bitwise XOR  (a^b), bitwise AND NOT (a&!b) and ternary operator (condition ? codeTrue : codeFalse).

Integers also have their separate set of operators for addition, subtraction and multiplication (division and modulo are sadly not implemented yet). They differ in a dot "." before the operator sign.

DSPC3 compiler is smarter than original code component. If all arguments of the operator are constants, the operator is evaluated during compilation and result is used as a constant instead. Some even provide few optimizations if one (or more) arguments are constants. This is also true for most functions.

a=1+2+3+2*2; //compiler automatically converts it to "a=10; a=b/4; //compiler converts it to a=b*0.25; which is cheaper on CPU.

## Functions

Functions from original Code Component are supported (NOTE: log10 was replaced with faster log2). However,  their assembler implementation is vastly superior. Also DSPC3 provides ability to customize them and add new functions (this is described in the end of the manual). Here is a table of basic functions currently present.

| to_i(float) | convert float to integer (rounding) |
| --- | --- |
| trunc(float) | convert float to integer (truncate) |
| to_f(int) | convert integer to float |
| rndint(float) | round float to nearest integer (returns float) |
| sin1(x), cos1(x), tan1(x) | trigonometic functions. Angle is provided in normalized form (1~2pi rad~360°) |
| abs(x) | absolute value (float only) |
| max(x,y) min(x,y) | maximum and minimum also works for integers if at least one of them is positive |
| pow(a,b) | a to the power of b |
| log2(x) | base 2 logarithm of x |
| min(x) max(x) | minimum/maximum of 4SSE channels (result is put into all the channels) |
| any(a) | bitwise OR of all 4 SSE channels |
| all(a) | bitwise AND of all 4 SSE channels |

more functions will be added over time...

## Code branching and loops

Code branching and loops is exceptionally awkward in stock Code Component. Although bitwise logic has exceptional CPU performance, its concepts are hard to grasp for many users. Reason why traditional IF statements as well as conditional loops are not present in original code component is, because they are insanely SSE incompatible. Nevertheless DSPC3 successfully implements them in SSE compatible way (at a price in CPU cycles though...). However, try to avoid them if possible.

Firstly, I'd like to mention few improvements to the bitwise logic. Newly added operators increase the flexibility significantly. The king of them all being...

### ternary operator

Ternary operator has 3 arguments. First one is the condition, second is an expression which's value is returned when condition is true and third is an expression which value is returned on false.  Unlike in C where ternary is logical, in DSPC3 it is bitwise. Also it is important to pinpoint that all 3 arguments are being executed. If assignment happens in any of them, it will always take effect.

//ternary operator condition ? CodeForTrue : CodeForFalse //it is a shortcut for { temp=condition; (CodeForTrue & temp)¦(CodeForFalse &! temp) } //useful when value needs to change only occasionaly a=(pulse) ? b : a; //sets "a" to value "b" on each pulse //useful when switching between two options happens c= (abs(in) > out) ? attack : release; //all 3 parts of the code are executed no matter the condition!!! c=b=0 a=(1<2) ? (b=3) : (c=6); //after: a=3 b=3 c=6 //ternary is bitwise 0xf0 ? 0x57 : 0x13 //returns 0x53

Ternary operator is very similar to IF statement, but it is significantly cheaper on CPU (it literally takes like 3cycles to perform - less than multiplication).

### IF statement

True solid IF statement is very hard to implement  in poly/mono4 compatible way. Nevertheless DSPC3 has such IF statement. From the user perspective, it does exactly what you'd expect. Evaluates the condition and executes code if true and not of false. With optional "else" statement... Unlike ternary operator, only one of the options is being executed and the other is skipped. Naturally, this happens for each SSE channel individually. It also returns the value of the executed code part (or zero on false, if "else" is not supplied.

//syntax if(condition) codeForTrue ; //returns zero on false if(condition) codeForTrue else codeForFalse; //be warned - everything after condition/else is considered the code a=if(1>2){3;}+1; //return 0 because "{3;}+1" is considered the code a=(if(1>2){3;})+1; //returns 1 //only the correct part is being executed c=b=0 a=if(1<2){b=3;} else{ c=6; }; //after: a=3 b=3 c=0 //may be used as a "smart hop" which only executes on value change streamin freq; streamin in; streamout out; float e=2.71828182846; float m2pi=-6.28318530; float pfreq=-1; float b1; if(freq!=pfreq){ //skips, if freq doesn't change pfreq=freq; b1=pow(e,m2pi*freq); //quite CPU heavy } out=(out-in)*a0+in; //iterate at sample-rate

Under the hood however, it works much more complicatedly then described. In reality, the code is executed if condition is true in any channel and else part is executed if any is false. That means sometimes both are physically being executed. What creates the impression of "channel selectivity" is, that assignment has different implementation when it's in IF statement.

if(condition) { a=b+c; } else{ x=pow(x,y); } //each "=" is being replaced with ternary. //Actual C-ish code looks more like: temp=condition; if( ~any(temp) ) goto else; a=temp ? (b+c) : a; if (~all(temp)) goto end; else: x=temp ? x : pow(x,y); end: //with nested IF statements it is even more complicated //because also previous conditions need to be taken into account

If CPU performance is also your goal, use IF statement only when you need to switch between complicated CPU heavy blocks of code. Irregular and hard to predict jumps may result in decreased CPU performance and poly/mono4-compatible IF statement uses exactly that type of code-jumping. For switching between smaller code segments, consider using ternary operator, perhaps even in form proposed above.

IF statement also has a "mono" version (called "ifm"). It only takes into consideration the result of condition SSEchannel 0. That means, truly only IF or ELSE part of the statements are executed (never both) and always the same in all 4 channels, thus resulting in much more SSE friendly assembler code.  Try to always use it in cases when condition is expected to have same result in all 4 channels.

//may be used as a "smart hop" which only executes on value change streamin freq; //frequency comes from green (for example knob) //thus is the same in all channels streamin in; streamout out; float e=2.71828182846; float m2pi=-6.28318530; float pfreq=-1; float b1; ifm(freq!=pfreq){ //skips, if freq doesn't change pfreq=freq; b1=pow(e,m2pi*freq); //quite CPU heavy } out=(out-in)*a0+in; //iterate at sample-rate

### while statement

Conditional loops were not possible in original Code Component, thus greatly limiting its possibilities. Like IF statement, they are not very SSE friendly. Currently only WHILE DO statement is implemented. It also returns the last value of the code from the last iteration. As you probably know,  conditional loops may result in infinite loops (which will crash Flowstone).

//syntax while(condition) code; //example: positive integer division streamin numerator; streamin denominator; streamout result; streamout remainder; int k=0,m=0; k=denominator; result=0; while(k<numerator) { k=k+denominator; result = result .+ 1i; }; remainder= k .- numerator;

Just like IF statements, WHILE statement has a mono version ( "whilem"). It only takes channel0 of the condition in the account.

### loop and hop statements

DSPC3 also contains original static "loop" and "hop". in each of them the argument must be positive integer (in case of "hop" it also must be power of two). They work exactly the same like in original Code Component. Loop returns the last value in its codeblock. Hop does too, but only when it is executed - when it's hopped, it may give off nonsense. This may get fixed in future updates.

# Function Library

DSPC3 comes with built in function library. It also allows you to add new functions, with a little skill in assembler and ruby. In future, libraries may be added from txt files on your hard-drive, simply by opening them in properties menu. Currently you have to copy-paste the functions into built in basic library.

Go inside the DSPC3 module > classes > main libraries

There is a text prim called "main function library". Inside are the "blueprints" of all functions that will be usable in DSPC3 - there you will paste your function "blueprint".

Following sections assume, you have basic knowledge about assembler and ruby.

## creating new function

First create new function by adding:

newFunction{ name=MyFunctionName; //specifies name of your function }

Note that the file-parsing does not support comments - using them may render all functions invalid or even crash Flowstone.

### input pattern

Now you have to specify for which argument types your function will be available. This is important, because they may exist multiple functions with the same name and they may require specific number and types of arguments.

When DSPC is compiling the code, it first compiles all the arguments, each returns a value. Then it creates a short string describing them and passes it to your function for validation. Argument may be either a variable "v", xmm register "r", constant "c" or array "a". Here are few examples:

streamin x; float y,mem[10]; MyFunction(x,y,5) //"vvc" -variable,variable,constant MyFunction(x,5*y) //"vr" (5*y) is an expression which returns its //result in xmm register MyFunction(mem,mem[0]) //"ar" -first argument is array //second is value from array (returned in xmm reg)

Instead of having to name all the possible combinations, DSPC3 allows you to specify Regexp object against which the input pattern will be compared to. In case you don't know about Regexp, here is a quick tutorial.

Regexp is short for "regular expression" it is compared against a string. Unlike string which is enclosed in quotation marks, Regexp is enclosed in /regexp/.

/vvc/ #matches any string containing "vvc" substring /vvc/==="cvvca" ->true /vvc/==="vrvc" ->false You may "anchor" begining of the Regexp to beginning of a string by putting ^ in the start /^vvc/==="cvvca" ->false /^vvc/==="vvccv" ->true Similarly end of Regexp may be anchored by $ /^vvc$/==="vvc" ->true - any other string would return false Often, you want some character to have multiple options. You may do this by listing them in [] brackets /^[avc]c$/==="ac" ->true /^[avc]c$/==="vc" ->true /^[avc]c$/==="rc" ->false You may also want a character to be optional. You do this by following it with ? /^av?c/==="ac" ->true /^av?c/==="avc" ->true Or you may want it to occur multiple times * - Zero or more times + - One or more times ? - Zero or one times (optional) {n} - Exactly n times {n,} - n or more times {,m} - m or less times {n,m} - At least n and at most m times /^c[rv]*$/=="c" ->true /^c[rv]*$/=="crvvrrr" ->true /^c[rv]*$/=="cvcr" ->false (because of the second c)

So, now you decided for which input patterns you function should be valid and add it into the blueprint:

newFunction{ name=MyFunctionName; inputs=/[rvc][rvc]v/; //two inputs, each being register, variable or //constant, followed by a variable }

### attributes

Now we have to specify some attributes.

complexity=1;

Complexity specifies the minimum number of xmm registers your function requires. This is important for the compiler, so it can free up the registers if necessary. Value should be anywhere from 0-8.

You may request higher number of registers if necessary during code compilation., but more on that later.

assigning=false; polyCompatible=false; comutative=false;

These are two optional attributes (they are false by default).

"assigning" attribute should be true if your function writes data into one of arguments (variables or arrays) or into some global variable. Using temporary variables does not count.

"polyCompatible" should be true if it writes data to memory in a poly-compatible way (more on that later). Compiler generally prefers " polyCompatible=false" functions (it assumes they are simpler) and uses "polyCompatible=true" only in IF and WHILE statements, because they need special care in this matter.

Assignment operator "=" is a good example of a "function", that has two implementations based on poly-compatibility.

"comutative" attribute specifies, that the order of arguments does not matter. "max(x,y)" is a good example of such function. When it is set to true, compiler may change the order of arguments so that xmm registers go first. Note: This has no effect on the order in which the arguments are executed (always left-to-right).

## Inserting code generator

Last thing we need to do is to specify the code generator, which will generate the assembler code for your function. Fortunately, compiler will handle most of the details, like putting in specific input names and changing the xmm registers to not clash with rest of the code.

You just have to add code genrator specification. This is a ruby codeblock, which should yield a assembler code as a string. The simplest thing you can do is to simply add the ASM code in quotation marks as follows:

newFunction{ name=MyFunctionName; inputs=/^[rvc][rvc]v$/; complexity=2; assigning=true; polyCompatible=false; code={'stage2; movaps xmm0,_0_; mulps xmm0,_1_; movaps _2_,xmm2; //return=xmm2' }; }

Now we have working assembler function. It multiplies first two arguments, writes the result into third argument (variable) and also returns that value.

## assembler code formating

As you can see, the assembler code requires to be formatted in specific way. Here are few rules:

### variable delcaration

First part is variable declaration. It works in very similar way to flowstone assembler. However, you must add additional info to the variable types, by preceding them with keyword:

const float one=1;

Declares a constant. The name of the variable is later changed, to match other constants with the same value. You must never write to these variables.

global float counter=0;

Declares a global variable. This variable will be available from outside the function, and it will be the same in all calls on the function. Use it to allow communication between individual occurrences. Note, that user may override the starting value.

local float in1=0;

Declares local variable. This variable will be specific to each occurrence of the function in the code. You may use it for example if your function is a filter, to remember previous inputs and outputs/states. Variable name will be changed during compilation to be unique.

temp float holdValue;

Declares a temporary variable. DSPC3 initializes a set of temporary variables, which may be used in specific parts of code to hold intermediate values. You can't rely on it to hold value form sample to sample, nor to have specific value to begin with, as it may be used by other parts of the code previously and after the function call. (Note: don't initialize it with a value!)

### code specification

stage0;

Marks the beginning of stage(0) code. This will be put at the very beginning of stage(0). Code in here may not refer to inputs and does not return any value. You may use it for example to initialize lookup tables.

stage2;

The actual code of the function (it may not end up in stage(2) specifically - that depends on where the function is called). Here you may reference individual arguments in form "_N_" where N is the argument index (starting from zero). If arguments are xmm registers, they will also have that matching number. Keep this in mind when writing your code, so you don't override their values before they can be used.

stage2; movaps xmm1,_0_; //here you have overwritten the content //of argument if it happened to be in xmm register addps xmm1,_1_; //if _0_ or _1_ are already registers, their index will also match movaps xmm0,_0_; movaps xmm1,_1_; //they may get compiled to something like movaps xmm6,xmm6; movaps xmm3,xmm3; //which is quickly noticed by the compiler and removed during //optimization phase

The result of your function should preferably end up in one of the xmm registers. Be aware though - always use registers with the least number. If your "complexity" attribute is set to 3, only use registers xmm0, xmm1 and xmm2. Otherwise you risk breaking expression evaluation and it may result in bugs that are quite hard to track down.

Note that the compiler may (and probably will) change their indexes in resulting compiled code.

Also, if your function is assigning and poly compatible, you may also use additional argument called "_POLY_". It references a temporary variable, that holds the combined bitmask  of all IF and WHILE statements.

//the asm code of poly compatible assignment operator. (whenever you //use "=" inside IF statement) //argument _0_ is the value, argument _1_ is the variable stage2; movaps xmm0,_0_; movaps xmm1,_POLY_; andps xmm0,xmm1; andnps xmm1,_1_; orps xmm0,xmm1; movaps _1_xmm0; //return=xmm0 //use similar structures when making poly-compatible versions of //your functions (this is necessary only for function that assign to //variables

stage3;

Marks the beginning of stage(3) code, which will be put at the end of stage3. Same rules apply as for stage(0) code.

//return=value

This specifies, which variable/register holds the result (references stage(2) code). Preferably it should be xmm register. Your function must always return some value, even if it's not meant to work that way. If your function is not meant to return anything, at least put "F0" there to return zero.

## Customizing code generator

As mentioned before, "code" attribute in the function definition is a ruby code. It is called by compiler and it is provided multiple parameters (as ruby variables).

| inputs | An array that holds the arguments. Variables, registers and arrays are presented as string objects, while constants are numbers (in integer or float format respectively) Note: they are listed in original order, before reordering happens (relevant if commutative attribute is set true) |
| --- | --- |
| regs | Array of xmm registers (again before reordering). This shouldn't concern you, just make sure you don't override it. |
| rn | number of available xmm registers. This is at minimum of what you specified in "complexity" attribute, but may be more. |
| poly | array of temporary variables that hold channel-specific flags form IF and WHILE statements. The last variable in the chain (poly[-1]) may be referenced in the asm code by "_POLY_". Others shouldn't normally be of any interrest. |
| @name,@ins,@code, @complexity@comutative, @assigning,@polyCompt | Instance variables that hold respective attributes of this function. |

Perhaps the best way to show you the options that it gives you, is to demonstrate an example. Let's say we want to make a "max" function, that takes any number of parameters - not just two.

newFunction{ name=max; inputs=/^[rvc][rvc]+$/; //regexp for at least two reg/var/const //inputs. Alternatively /^[rvc]{2,}$/ complexity=1; //we may change this later if we need more than one assigning=false; polyCompatible=false; }

As you know, the number of "maxps" opcodes will depend on the number of inputs now. So writing a plain string for the code is not possible. Instead, we measure the number inputs we will iterate trough inputs:

newFunction{ name=max; inputs=/^[rvc][rvc]+$/; complexity=1; code={ cod="stage2;\n" inputs.each_index{¦i¦ cod+= if i==0 #we need "movaps" for first one "movaps xmm0,_" + i.to_s + "_;\n" else "maxps xmm0,_"+ i.to_s +"_;\n" end } cod+"//return=xmm0"};}

This will generate ASM code that will depend on number of inputs.

You can also make the code much more efficient. Suppose some of the inputs are constants. You don't have to calculate their maximum at sample rate - you may pre-compile them into one constant, because you know their values. We will take advantage of fact, that constant arguments are passed as numbers, not strings.

(from now on, I will only write the content of code={};)

#collect all inputs that are constant, and calculate their maximum mx=inputs.collect{¦ins¦ ins.is_a?(Numeric)}.max cod="const float mx="+mx.to_s+";\n stage2;\n" first=true #this will become false once first non-constant #input is used inputs.each.with_index{¦ins,i¦ if (ins.is_a?(String)) #skip inputs that are constants if first cod+="movaps xmm0,_"+i.to_s+"_; "maxps xmm0,mx;\n" else cod+="maxps xmm0,_"+i.to_s+"_;\n" end end first=false end } cod+"//return=xmm0"

The slight issue is, "maxps" is a floating point operation. If integer is supplied, its binary structure is used as a float. However, our constant input pre-processing doesn't take that into account. We may fix that by converting integers to floats using pack/unpack.

mx=inputs.collect{¦ins¦ ins.is_a?(Numeric)}.map{ ¦val¦ if val.is_a?(Integer) [val].pack("L").unpack("f")[0] else val end}.max

The obvious question to ask is: "what if all arguments are constants?" In that case you don't really need to generate any assembler code at all - you just have to return a constant.

There is a quick way to do that - simply use "return" to prematurely end the full code-generation process. Returning the constant.

mx=inputs.collect{¦ins¦ ins.is_a?(Numeric)}.map{ ¦val¦ if val.is_a?(Integer) [val].pack("L").unpack("f")[0] else val end}.max if inputs.all?{¦ins¦ ins.is_a?(Numeric) return mx end cod="const float mx="+mx.to_s+";\n stage2;\n" first=true #this will become false once first non-constant #input is used inputs.each.with_index{¦ins,i¦ if (ins.is_a?(String)) #skip inputs that are constants if first cod+="movaps xmm0,_"+i.to_s+"_; "maxps xmm0,mx;\n" else cod+="maxps xmm0,_"+i.to_s+"_;\n" end end first=false end } cod+"//return=xmm0"

No code will be generated in that case - just a constant will be used in place of the function call. This is very convenient, for code optimization in DSPC3. Most operators and functions have this kind of "back door", which allows you to write both readable and efficient code.

Speaking of efficiency, our "max" function implementation is not optimal. We are calculating maximum recursively in series. This prevents our computer to do parallel operations. finding maximum using tree-like structure is faster. However, it uses more xmm registers.

Problem is, because we do not know how many arguments the function will have, we have no way of predicting how many registers we need.

Luckily we know exactly how many xmm-registers we have available ( the "rn" variable). We can write "rn" parallel "maxps" chains and then find maximum of them. Also, if some of the arguments are in xmm registers, they will have matching index.

(note: in this case we're not filtering out constant arguments - code would be much more complicated)

cod="const float mx="+mx.to_s+";\n stage2;\n" #first we generate par=[rn,inputs.length].min sg=(inputs.length.to_f/par).ceil-1 (0...par).each{¦i¦ cod+="movaps xmm"+i.to_s+",_"+i.to_s+"_;\n" } (1..sg).each{¦j¦ (0...par).each{¦i¦ cod+="maxps xmm"+i.to_s+",_"+(j*par).to_s+"_;\n" } } pars=(0...par).to_a while pars.length>1 pars2=pars.each_slice(2).map{ ¦i,j¦ if j then cod+="maxps xmm"+i.to_s+",xmm"+j.to_s+";\n" end i} pars=pars2 end cod+"//return=xmm0"