Quilcom STAGE

(STereo Audio Gets Enhanced!)

Design

The Quilcom STAGE is a plugin which hosts 9 processing modules each dedicated to enhancing a stereo mix to give added width, localisation, vibrancy and depth. Various techniques are used to achieve this. Of course it can also be applied to individual stems provided the source is in stereo.

The user can audition each processor in turn, find the sweet spot for its settings, and then do A/B comparisons without interrupting the audio source. Only the selected processor will use CPU cycles and the DAW will recall the settings and selection with the song project file.

Operation

At the top of the front panel is the processor selection and you can click the main body of the dropdown menu or use the arrow buttons to page through the effects. Only the chosen processor will be heard.

Each processor will have a few knobs to adjust the parameters and hopefully their labels will indicate the function.

Not that each processor has a volume control. Usually this can be set about midway but can be adjusted to give similar apparent volume level when doing A/B comparisons to get a more accurate impression of the results.

Above the level bar graph meters (with clip-hold LEDs) is the bypass LED-switch which bypasses the processors, for comparison purposes.

To the right of the meters is a goniometer which helps to visualise the phase relationships between the two stereo channels. To get a well-balanced sound stage, should you want that, this display can help to get the ideal circular shape and may be more accurate than your ears, speakers and listening conditions.

Below the goniometer there is the overall output volume and a balance control. The balance control will affect the goniometer display and is useful for offsetting any unbalanced effect of any processor.

There are no presets because the settings will ultimately be based on what you have and what you wish to achieve, and this will probably differ between each application. However the DAW interfacing will allow you to save the settings on a per-song basis and automate as desired since each control has a unique parameter name.

Notes

In 1990 John Bedini registered a patent for creating an enhanced stereo experience from a conventional mix. The “Bedini patented” processor in the STAGE is modelled exactly on the analogue circuitry of that patent.

Later he manufactured a product he called the B.A.S.E. but there is no circuit available and it clearly differed to his original patented design.

There have been variations of this product released since the FIRST. From the little information and sounds I have been able to get I’ve made an approximation to what I would expect to find inside the box. Apparently there are several potted circuit elements inside so reverse engineering isn’t practical, especially considering  the original sold for $6000!

My SECOND B.A.S.E. is a small variation on the FIRST, based on careful listening to some recordings sent to me by Marian Brezovan who owns a FIRST B.A.S.E.

There are very mixed reviews and comments online about this product, ranging from truly wonderful to being an expensive scam. My versions of the process at least produce some good results in my opinion, and the main thing is that it doesn’t cost $6000!